The Getaway  (50004)

FUSE LIST

| POWER DRIVER BOARD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| F101 | +50V DC General (Left Flipper) | 3A, 250V, SB Not Used |  |  |  |
| F102 | +50V DC General (Right Flipper) | 3A, 250V, SB Not Used |  |  |  |
| F103 | Solenoids #25 to #28 | 3A, 250V, SB |  |  |  |
| F104 | Solenoids #9 to #16 | 3A, 250V, SB |  |  |  |
| F105 | Solenoids #1 to #8 | 3A, 250V, SB |  |  |  |
| F106 | G.I. #5 White-Violet | 5A, 250V, SB |  |  |  |
| F107 | G.I. #4 White-Green | 5A, 250V, SB |  |  |  |
| F108 | G.I. #3 White-Yellow | 5A, 250V, SB |  |  |  |
| F109 | G.I. #2 White-Orange | 5A, 250V, SB |  |  |  |
| F110 | G.I. #1 White-Brown | 5A, 250V, SB |  |  |  |
| F111 | Flasher Secondary | 5A, 250V, SB |  |  |  |
| F112 | Solenoid Secondary | 7A, 250V, SB |  |  |  |
| F113 | +5V Logic | 5A, 250V, SB |  |  |  |
| F114 | +18V Lamp Matrix | 8A, 32V, N.B. |  |  |  |
| F115 | +12V Switch Matrix | 3/4A, 250V, FB |  |  |  |
| F116 | +12V Secondary | 3A, 250V, SB |  |  |  |
| Audio Board |  |  |  |  |  |
| F501 | -25V Circuit | 3A, 250V, SB |  |  |  |
| F502 | +25V Circuit | 3A, 250V, SB |  |  |  |
| Dot Matrix Controller Board |  |  |  |  |  |
| F601 | +62V Circuit | 3/8A, 250V, FB | Fliptronic II Controller Board |  |  |
| F602 | -113V & -125V Circuits | 3/8A, 250V, FB | F901 | Upper Right Flipper | 3A, 250V, SB |
| LINE FILTER |  |  | F902 | Upper Left Flipper | 3A, 250V, SB |
| Foreign |  | 5A, 250V, SB | F903 | Lower Right Flipper | 3A, 250V, SB |
| Domestic |  | 8A, 250V, SB | F904 | Lower Left Flipper | 3A, 250V, SB |

SOLENOID/FLASHER TABLE

| Sol No. | Function | Solenoid Type | Voltage Connections |  |  |  | Drive Xistor | Drive Connections |  |  | Drive Wire Color | Solenoid Part Number Flashlamp Type |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Playfield | Backbox | Cabinet |  |  | Playfield | Insert | Cabinet |  | Playfield |  | Insert |
| 01 | Diverter High | High Power | J107-3 |  |  |  | Q82 | J130-1 |  |  | VIO-BRN | A-14701 |  |  |
| 02 | Up Ramp | High Power | J107-3 |  |  |  | Q80 | J130-2 |  |  | VIO-RED | AE-26-1200 |  |  |
| 03 | Down Ramp | High Power | J107-3 |  |  |  | Q78 | J130-4 |  |  | VIO-ORG | SM1-28-900-DC |  |  |
| 04 | Locker (Disappearing Post) | High Power | J107-3 |  |  |  | Q76 | J130-5 |  |  | VIO-YEL | AE-26-1200 |  |  |
| 05 | Left Slingshot | High Power | J107-3 |  |  |  | Q64 | J130-6 |  |  | VIO-GRN | AE-26-1500 |  |  |
| 06 | Right Slingshot | High Power | J107-3 |  |  |  | Q66 | J130-7 |  |  | VIO-BLU | AE-26-1500 |  |  |
| 07 | Knocker | High Power |  | J107-3 |  |  | Q68 | J130-8 |  |  | VIO-BLK | AE-23-800 |  |  |
| 08 | Kickback | High Power | J107-3 |  |  |  | Q70 | J130-9 |  |  | VIO-GRY | AE-23-800 |  |  |
| 09 | Eject Hole | Low Power | J107-2 |  |  |  | Q58 | J127-1 |  |  | BRN-BLK | AE-26-1200 |  |  |
| 10 | Diverter Low | Low Power | J107-3 |  |  |  | Q56 | J127-3 |  |  | BRN-RED | A-14701 |  |  |
| 11 | Ball Release (Trough) | Low Power | J107-2 |  |  |  | Q54 | J127-4 |  |  | BRN-ORG | AE-27-1200 |  |  |
| 12 | Plunger Kicker | Low Power | J107-2 |  |  |  | Q52 | J127-5 |  |  | BRN-YEL | A-14789 |  |  |
| 13 | Top Jet Bumper | Low Power | J107-2 |  |  |  | Q50 | J127-6 |  |  | BRN-GRN | AE-26-1200 |  |  |
| 14 | Left Jet Bumper | Low Power | J107-2 |  |  |  | Q48 | J127-7 |  |  | BRN-BLU | AE-26-1200 |  |  |
| 15 | Bottom Jet Bumper | Low Power | J107-2 |  |  |  | Q46 | J127-8 |  |  | BRN-VIO | AE-26-1200 |  |  |
| 16 | Outhole | Low Power | J107-2 |  |  |  | Q44 | J127-9 |  |  | BRN-GRY | AE-26-1200 |  |  |
| 17 | Right Bank Flasher | Flasher | J107-6 |  |  |  | Q42 | J126-1 |  |  | BLK-BRN | #89 |  | #906 |
| 18 | Supercharger Flasher | Flasher | J107-6 |  |  |  | Q40 | J126-2 | J125-2 |  | BLK-RED |  |  | #906 |
| 19 | Left Slingshot Flasher | Flasher | J107-6 |  |  |  | Q38 | J126-3 | J125-3 |  | BLK-ORG | #89 |  | #906 |
| 20 | Free Ride Flasher | Flasher | J107-6 |  |  |  | Q36 | J126-4 |  |  | BLK-YEL |  |  | #906 |
| 21 | Left Ramp Flasher | Flasher | J107-6 |  |  |  | Q28 | J126-5 | J125-6 |  | BLU-GRN | #89 |  | #906 |
| 22 | Left Bank Flasher | Flasher | J107-6 |  |  |  | Q30 | J126-6 |  |  | BLU-BLK | #89 |  | - |
| 23 | Flipper Flasher | Flasher | J107-6 |  |  |  | Q34 | J126-7 | J125-8 |  | BLU-VIO | #89 |  | #906 |
| 24 | Right Slingshot Flasher | Flasher | J107-6 |  |  |  | Q32 | J126-8 | J125-9 |  | BLU-GRY | #89 |  | #906 |
| 25 | Enable 1 | Low Power | J107-1 |  |  |  | Q26 | J122-1 |  |  | BLU-BRN | A-15685 |  |  |
| 26 | Enable 2 | Low Power | J107-1 |  |  |  | Q24 | J122-2 |  |  | BLU-RED | A-15685 |  |  |
| 27 | Revolving Lamp | Low Power |  |  | J106-5 |  | Q22 |  |  | J123-4 | BLU-ORG | 14-7971 |  |  |
| 28 | Enable 3 | Low Power | J107-1 |  |  |  | Q20 | J122-4 |  |  | BLU-YEL | A-15685 |  |  |
| General Illumination |  |  | Voltage Connections |  |  |  | Drive Xistor | Drive Connections |  |  | Drive Wire | Bulb Type |  |  |
|  |  |  | Playfield | Backbox | Cabinet |  |  | Playfield | Backbox | Cabinet |  | Playfield | Backbox |  |
| 01 | Playfield G.I. | G.I. | J120-1 |  |  |  | Q18 | J120-7 |  |  | WHT-BRN | #555 | #555 |  |
| 02 | Playfield G.I. | G.I. | J120-2 |  |  |  | Q10 | J120-8 |  |  | WHT-ORG | #555 | #555 |  |
| 03 | Insert G.I. | G.I. | J120-3 | J121-3 |  |  | Q14 |  | J121-9 |  | WHT-YEL | #555 | #555 |  |
| 04 | Insert G.I. | G.I. |  | J121-5 |  |  | Q16 |  | J121-10 |  | WHT-GRN |  | #555 |  |
| 05 | Insert G.I. | G.I. |  | J121-6 | J119-3 |  | Q12 |  | J121-11 | J119-1 | WHT-VIO |  | #555 |  |
| Flipper Circuits |  |  | Playfield Voltage Connection |  | Drive Transistors |  |  | Playfield Drive |  | Drive Wire Colors |  | Coil Part No. | Coil Colors |  |
|  |  |  |  |  | Power | Hold |  | Connections |  | # Power Hold |  |  |  |  |
|  | LOWER LEFT FLIPPER | Flipper | J907-6, 7 (GRY-YEL) |  | Q3 | Q9 |  | J902-7, 9 |  | BLU-GRY ORG-BLU |  | FL-15629 | BLUE |  |
|  | LOWER RIGHT FLIPPER | Flipper | J907-8, 9 (BLU-YEL) |  | Q4 | Q11 |  | J902-11, 13 |  | BLU-VIO ORG-GRN |  | FL-15629 | BLUE |  |
|  | UPPER RIGHT FLIPPER | Flipper | J907-4, 5 (BLU-YEL) |  | Q2 | Q7 |  | J902-4, 6 |  | BLK-BLU ORG-VOI |  | FL-11630 | RED |  |

LAMP MATRIX	Red 	Yellow (B+)	16-10213

| Column Row | 1 Yellow- Brown J137-1 Q98 | 2 Yellow- Red J137-2 Q97 | 3 Yellow- Orange J137-3 Q96 | 4 Yellow- Black J137-4 Q95 | 5 Yellow- Green J137-5 Q94 | 6 Yellow- Blue J137-6 Q93 | 7 Yellow- Violet J137-7 Q92 | 8 Yellow- Gray J137-8 Q91 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 Red- Brown J133-1 Q90 | FREEWAY 1 11 | 2X 21 | TOP RED 31 | TACH 1 41 | SHOOT AGAIN 51 | RIGHT RETURN LANE 61 | 4TH GEAR 71 | TACH 9 81 |
| 2 Red- Black J133-2 Q89 | FREEWAY 2 12 | 4X 22 | TOP YELLOW 32 | TACH 2 42 | KICKBACK 52 | LEFT RETURN LANE 62 | 5TH GEAR 72 | TACH 10 82 |
| 3 Red- Orange J133-4 Q88 | FREEWAY 3 13 | HOLD BONUS 23 | TOP GREEN 33 | TACH 3 43 | TACH 11 53 | SIX BANK BOTTOM 63 | STOP LIGHT RED 73 | MIDDLE RED 83 |
| 4 Red- Yellow J133-5 Q87 | FREEWAY 4 14 | 6X 24 | RIGHT FREEWAY 34 | TACH 4 44 | TACH 12 54 | SIX BANK MIDDLE 64 | STOP LIGHT YELLOW 74 | MIDDLE YELLOW 84 |
| 5 Red- Green J133-6 Q86 | FREEWAY 5 15 | 8X 25 | SPECIAL 35 | TACH 5 45 | TACH 13 55 | SIX BANK TOP 65 | STOP LIGHT GREEN 75 | MIDDLE GREEN 85 |
| 6 Red- Blue J133-7 Q85 | SPEED 16 | GET AWAY 26 | VIDEO MODE 36 | BOTTOM RED 46 | TACH 14 56 | SUPER CHARGER 66 | 1ST GEAR 76 | TACH 6 86 |
| 7 Red- Violet J133-8 Q84 | LEFT FREEWAY 17 | SPEED MILLIONS 27 | RANDOM LAMP 37 | BOTTOM YELLOW 47 | TACH 15 57 | RED LINE MANIA 67 | 2ND GEAR 77 | TACH 7 87 |
| 8 Red- Grey J133-9 Q83 | LOCK 18 | SUPER JACKPOT 28 | EXTRA BALL 38 | BOTTOM GREEN 48 | SHIFT 58 | START BUTTON 68 | 3RD GEAR 78 | TACH 8 88 |

J1XX = Power Driver Board

SWITCH MATRIX	White	Green

| DEDICATED GROUNDED SWITCHES |  |  |  | Column Row | 1 Green- Brown J207-1 U20-18 | 2 Green- Red J207-2 U20-17 | 3 Green- Orange J207-3 U20-16 | 4 Green- Yellow J207-4 U20-15 | 5 Green- Black J207-5 U20-14 | 6 Green- Blue J207-6 U20-13 | 7 Green- Violet J207-7 U20-12 | 8 Green- Grey J207-9 U20-11 | FLIPPER GROUNDED SWITCHES |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Orange-Brown J205-1 LEFT COIN CHUTE U17-5 D1 |  |  |  | 1 White- Brown J209-1 U18-11 | NOT USED 11 | SLAM TILT 21 | LEFT SLING 31 | TOP YELLOW 41 | TOP GREEN 51 | TOP JET 61 | TOP LOOP 71 | OPTO 1 81 | BLACK-GREEN J906-1 LOWER RIGHT E.O.S. SWITCH F1 |
| Orange-Red J205-2 CENTER COIN CHUTE U17-7 D2 |  |  |  | 2 White- Red J209-2 U18-9 | NOT USED 12 | COIN DOOR CLOSED 22 | RIGHT SLING 32 | MIDDLE YELLOW 42 | MIDDLE GREEN 52 | LEFT JET 62 | MIDDLE LOOP 72 | OPTO 2 82 | BLUE-VIOLET J905-1 LOWER RIGHT FLIPPER BUTTON F2 |
| Orange-Black J205-3 RIGHT COIN CHUTE U17-11 D3 |  |  |  | 3 White- Orange J209-3 U18-5 | START BUTTON 13 | TICKET OPTO. 23 | GEAR SHIFTER LOW 33 | BOTTOM YELLOW 43 | BOTTOM GREEN 53 | BOTTOM JET 63 | BOTTOM LOOP 73 | OPTO 3 83 | BLACK-BLUE J906-3 LOWER LEFT E.O.S. SWITCH F3 |
| Orange-Yellow J205-4 4TH COIN CHUTE U17-9 D4 |  |  |  | 4 White- Yellow J209-4 U18-7 | PLUMB BOB TILT 14 | ALWAYS CLOSED 24 | GEAR SHIFTER HIGH 34 | RIGHT BANK BOTTOM 44 | RAMP DOWN 54 | NOT USED 64 | TOP LOCK 74 | OPTO MADE LOOP 84 | BLUE-GRAY J905-2 LOWER LEFT FLIPPER BUTTON F4 |
| Orange-Green J205-6 |  |  |  | 5 White- Green J209-5 U19-11 | LEFT FREEWAY BOTTOM 15 | LEFT OUTLANE 25 | NOT USED 35 | RIGHT BANK MIDDLE 45 | OUTHOLE 55 | MADE UP/DOWN RAMP 65 | MIDDLE LOCK 75 | ENTER LEFT RAMP 85 | BLACK-VIOLET J906-4 UPPER RIGHT E.O.S. SWITCH F5 |
| Normal Function Srv Crdts | Test Function Escape |  |  |  |  |  |  |  |  |  |  |  |  |
| D5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Orange-Blue J205-7 |  |  |  | 6 White- Blue J209-7 U19-9 | LEFT FREEWAY TOP 16 | LEFT FLIPPER LANE 26 | TOP RED 36 | RIGHT BANK TOP 46 | LEFT TROUGH 56 | NOT USED 66 | BOTTOM LOCK 76 | LEFT BANK BOTTOM 86 | BLACK-YELLOW J905-3 UPPER RIGHT FLIPPER BUTTON F6 |
| Normal Function Volume Dn |  | Test Function Down |  |  |  |  |  |  |  |  |  |  |  |
| D6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Orange-Violet J205-8 |  |  |  | 7 White- Violet J209-8 U19-5 | RIGHT FREEWAY BOTTOM 17 | RIGHT FLIPPER LANE 27 | MIDDLE RED 37 | NOT USED 47 | CENTER TROUGH 57 | MADE LEFT RAMP 67 | EJECT HOLE 77 | LEFT BANK MIDDLE 87 | BLACK-GRAY J906-5 UPPER LEFT E.O.S. SWITCH F7 |
| Normal Function Volume Up |  | Test Function Up |  |  |  |  |  |  |  |  |  |  |  |
| D7 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Orange-Grey J205-9 |  |  |  | 8 White- Grey J209-9 U19-7 | RIGHT FREEWAY TOP 18 | RIGHT OUTLANE 28 | BOTTOM RED 38 | NOT USED 48 | RIGHT TROUGH 58 | NOT USED 68 | SHOOTER 78 | LEFT BANK TOP 88 | BLACK-BLUE J905-5 UPPER LEFT FLIPPER BUTTON F8 |
| Normal Function Begin Test |  |  | Test Function Enter |  |  |  |  |  |  |  |  |  |  |
| D8 |  |  |  |  |  |  |  |  |  |  |  |  |  |

J2XX = CPU Board	= OPTO, TYPICALLY CLOSED